Polara Quick Checklist

Start up:

- Check logbook and microscope to locate all the cartridges

- Refill the liquid nitrogen

- Collect new gain reference for K2 in Digital Micrograph using beam settings for ‘Exposure’ mode (see [Instructions for Gatan K2 Camera])

Loading specimen:

- Be careful with the clip ring handling tool, especially when it is cold

- Cold down the cold trap pump thoroughly before use

- Make sure the tweezers are closed before grabbing cartridge from MSC

- In every step check the cartridge to see if it is in right loaction

Shut down:

- Close the column valves

- Remove the cartridge from the column and put it back into the MSC, only for those who know how and are allowed to do so

- Remove the dewars from the MSC and insertion rod, empty them and place them upside down into the Styrofoam container

- Clean up and organize the sample loading area

- Refill the liquid nitrogen dewar on the microscope

- Log out of your account and log into guest account

  - Username: polara

  - Password: polara

- Transfer all of your images from the GPU to your own account/computer

- Heat the K2 to 50°C, set the time to as much as permitted, leave one hour for K2 cooling down to -20°C before next user start

- Sign the log book and note any problems

If you encounter any problems, or think anything is strange with the microscope, please contact Zongli or Sarah before continuing.